Summary of the results of the CFA and EPA Carbon monoxide (CO) monitoring in Morwell

Qualifying information

- The data presented in this summary is preliminary data that came directly from CFA and EPA’s air monitoring sites. This data may be adjusted at a later time through a standardised procedure to allow for instrument errors, power interruptions and the like.

- The CFA and EPA instruments are different and the results may not be directly comparable. Further analysis to understand the relationship between the data measured by CFA and EPA instruments will be done at a later date.

Key points

- EPA has been monitoring Carbon Monoxide (CO) with good data capture at Morwell East since February 19 and at Morwell South since February 21.

- CO levels have been measured at 6 sites by the CFA since Feb 12, but there are some gaps in the data. The best data capture from the CFA sites are at Sacred Heart (primary school) and Morwell Police Station.

- The NEPM ambient air quality standard for CO is an 8 hour average of 9 ppm (parts per million). More information about the NEPM (ambient air quality) can be found at the following link (http://www.scew.gov.au/nepms/ambient-air-quality).

- 8 hour average CO levels have been measured in excess of the 9 ppm NEPM air quality standard at the EPA monitoring station in Morwell South on 3 of the 14 days since EPA began monitoring for CO at that location.

- 8 hour average CO levels have not breached the 9 ppm NEPM air quality standard at the EPA monitoring station in Morwell East since EPA began monitoring for CO at that location on Feb 19.

- The readings from the CFA instruments are not suitable for assessing compliance against air quality standards in the National Environment Protection Measure (Ambient Air Quality), however they can still be used to give an indicative guide to CO levels.

- Measurements by CFA instruments indicate CO levels were very high in the southern area of Morwell for the period from midday on Saturday Feb 15 to midnight on Sunday Feb 16. During this period, 8 hour average CO levels between 20-35 ppm were recorded at the Morwell Bowls Club and near the corner of Keegan and Wallace Streets. 8 hour average CO levels peaked at about 20 ppm at Sacred Heart (primary school) and 2 Kerrie Street.

- 8 hour average CO levels have not been recorded above 9 ppm at any of the CFA or EPA monitoring locations since February 27.

Table 1. 8 hour average CO levels of greater than 9ppm have been recorded at the following locations on the following days

| Location | Number of days when 8 hour average CO levels have peaked at greater than 9ppm | Days when 8 hour average CO levels have peaked at greater than 9ppm |
| --- | --- | --- |
| (EPA) Morwell East | 0 |  |
| (EPA) Morwell South | 3 | Feb 21, 22, 26 |
| (CFA) 2 Kerrie St | 2 | Feb 16, 21 |
| (CFA) Bowls Club | 3 | Feb 16, 17 21 |
| (CFA) cnr Keegan and Wallace St | 5 | Feb 15, 16, 22, 26, 27 |
| (CFA) Maryvale Crescent child centre | 2 | Feb 16, 22 |
| (CFA) Morwell Police Station | 1 | Feb 16 |
| (CFA) Sacred Heart (primary school) | 2 | Feb 16, 17 |

The map below shows the locations where CO has been monitoring by EPA (green pins) and the CFA (yellow pins)

This image was constructed using Google Earth.

The graphs below are plots of 8-hour rolling average Carbon Monoxide (CO) data at CFA and EPA sites since Feb 12.